Technical Bulletin

TITLE:	DCS T1 Card Update 				TB    9802

DATE:	May 13, 1998			      		NO. OF PAGES:	1 of 1

ISSUED BY:	TECHNICAL SUPPORT DEPARTMENT

There have been incidents where the DCS T-1 card causes noise throughout a multi-cabinet system when the 4th slot of the expansion cabinet(s) is used.

This noise can be eliminated by ensuring that the EXPN B card(s) is the current revision, containing a T1NM modification.

All EXPN B cards shipped as of  01/05/98 have been modified.  All modified cards will have a “T1NM” label next to the serial number.

Any dealer experiencing the unusual noise throughout a multi-cabinet DCS equipped with T-1 should first determine if slot 4 of any expansion cabinet is in use.  Then determine if the EXPN B card(s) contains the “T1NM”  label.

If you find that a modified EXPN B card is needed, contact Samsung Technical Support at (800)737-7008.