Technical Bulletin

TITLE:	DCS Compact MISC Card		               BULLETIN NO.:TB 9901

DATE:	February 18, 1999					     NO. OF PAGES:	1 of 1

ISSUED BY:	TECHNICAL SUPPORT DEPARTMENT 	    REPLACES TB 9804

There is some confusion regarding the serial cable connections on the MISC and MISC2 cards in the DCS Compact / DCS 50si

The diagrams in the DCS Compact Installation Section, figures 7-8 and 7-9, dated June 1995, should be changed to reflect the following pin connections:

The diagrams in the DCS 50si Installation Section, figures 7-8 and 7-9, dated December 1998, should be changed to reflect the following pin connections:

Figure 7-8

PIN CONNECTIONS FOR MISC CARD TO PERSONAL COMPUTER

MISC CARD(DB9)		PC (DB9)	PC (DB25)

TXD  2			PIN 2		PIN 3

RXD 3				PIN 3		PIN 2

GND 7				PIN 5		PIN 7

DTR 5				PIN 4		PIN 20

Figure 7-9

PIN CONNECTIONS FOR MISC CARD TO MODEM

MISC CARD(DB9)		MODEM (DB25)

TXD  2			PIN 2	TXD

RXD 3				PIN 3	RXD

GND 7				PIN 7	GND

PIN 6   DSR

PIN 20	 DTR