ELECTRONICS

##### TECHNICAL BULLETIN

#### Date: April 1, 2001

###### NO.:	2001-09

##### TITLE: DCS Compact database conversion to add the SVMi-4 card to an existing system

This bulletin demonstrates the upgrade procedure for existing DCS Compact systems to use the SVMi-4 VMAA card. By accomplishing the conversion, installation time will be reduced. The DCS Compact database may be converted from software version 1.x or 2.x to software version 2.5. This procedure covers the Basic and CID software packages.

You will need the following items to complete the database conversion:

MISC 1 or MISC 2 card

PCMMC = pc_0415.exe     [version 1.x]

PCMMC = pc240128.exe   [version 2.x]

PCMMC = pccomp25.exe  [version 2.5]

DCS Compact version 1.x to 2.5

The database conversion is a three step process. First the database must be downloaded using PCMMC version pc_0415. Then convert the database using PCMMC version pc240128, this brings the database from version 1.X to version 2.0. Next, convert the database from version 2.0 to 2.5 using PCMMC version pccomp25. After the conversion, replace the Compact software and add the required hardware. Then upload the converted database into the DCS Compact.

The following steps will walk you through the procedure to complete the conversion.

- Download the database using PCMMC version pc_0415

- Name the database [i.e. abcdef12.dcs]
****the file names must be different, use the version number as the last two characters of the file name****

- Copy the database (abcdef12.dcs) to the directory for PCMMC version pccomp25

- Go to the directory for pccomp25

- Type “converdb”

- On the screen it will show: 
 	‘Enter backup DB file name….”filename.dcs” [i.e. abcdef12.dcs]
 	then press ‘enter’

- On the screen it will show:
 	‘Enter output file name….”filename.dcs” [i.e. abcdef20.dcs]
 	then press ‘enter’
****the file names must be different, use the version number as the last two characters of the file name****

- On the screen it will show:
 	 ‘conversion process is successful’

- Type “convto25”

- On the screen it will show:
 	‘Enter backup DB file name….”filename.dcs”  [i.e. abcdef20.dcs]
 	then press ‘enter’

- On the screen it will show:
 	‘Enter output file name….”filename.dcs”  [i.e. abcdef25.dcs]
 	then press ‘enter’
****the file names must be different, use the version number as the last two characters of the file name****

- On the screen it will show:
 	 ‘conversion process is successful’

- Change the DCS Compact software to version 2.5 (4 eproms)

- Upload the database [i.e. abcdef25.dcs]

- Complete programming as required for SVMi-4 operation

DCS Compact version 2.x to 2.5

The database conversion is a two step process. First download the database using PCMMC version pc240128. Then convert database version 2.0 – 2.4 to version 2.5 using PCMMC version pccomp25. After the conversion, replace the software and add the required hardware. Then upload the converted database into the DCS Compact.

- Download the database using PCMMC version pc240128

- Name the database [i.e. abcdef24.dcs]
****the file names must be different, use the version number as the last two characters of the file name****

- Copy the database (abcdef24.dcs) to the directory for PCMMC version pccomp25

- Go to the directory for pccomp25

- Type “convto25”

- On the screen it will show:
 	‘Enter backup DB file name….”filename.dcs”  [i.e. abcdef20.dcs]
 	then press ‘enter’

- On the screen it will show:
 	‘Enter output file name….”filename.dcs”  [i.e. abcdef25.dcs]
 	then press ‘enter’
****the file names must be different, use the version number as the last two characters of the file name****

- On the screen it will show:
 	 ‘conversion process is successful’

- Change the DCS Compact software to version 2.5 (4 eproms)

- Upload the database [i.e. abcdef25.dcs]

- Complete programming as required for SVMi-4 operation

If you have any questions about this or any other Technical bulletins please contact Technical support by phone 1800 737 7008 or by email techsupportmiami@sta.samsung.com.